Homework 2

NYCU Computer Vision 2024 Spring

Student ID : 312605001 / Student Name : 歐庭維

- Image Stitching

- Base task

Stitch 2 image

Stitch 3 image

- Challenging task

- My Complementation

- Stitching Pipline

- SIFT

I use OpenCV “SIFT_create” to get the key point position and its descriptor in this part.

- KNN

In my first trial, I use “cv.BFMatcher / knnMatch” to find the same kp between two image.

I also tried to implement the knn matcher by myself. The threshold remains the same, and the output stitched image look the same.

- Homography matrix

After obtaining good matches, the next step is to calculate the homography matrix between two images. It's important to remember to standardize the homography matrix

before returning it.

- RANSAC

Due to numerous misaligned pairs, I resort to using RANSAC (Random Sample Consensus) to find the best homography matrix. After conducting several trials, I found that setting the parameters "ransac_threshold=5" and "iteration=1000" consistently yields stable results in each case.

- Cylinder projection

After many trial, I have determined that setting the focal length to 4000 produces the best results in this particular context.

| Focal = 1200 | Focal = 2200 |
| --- | --- |

- Crop image

I stitch each image from left to right, so the left image will be distorted. So I crop the distorted part after stitching each images.

- Remove Black Border

To address the sharp edge at the jointed place when stitching two images together, I employ the "medianBlur" technique to smooth out the transition and create a more seamless blend.

- Stitching IMG

In this part, I use “cv.warpPerspective” to transfer the image by the best homography matrix and stitching the left and right image together.

- Discuss different blending method result.

- Considering the Brightness

I noticed significant differences in the ISO values in the challenge task, which could lead to abnormal stitching results. Therefore, before stitching each image, I calculate the brightness of each one and find the average brightness. Then, I calibrate each image to the same brightness to address this issue.

| Considering Brightness | Without Considering Brightness |
| --- | --- |

- Distorted issue in the base case

Since I stitched the first two photos together before overlaying them with the others, it causes the leftmost image (the first one) to be continuously affected by the homography matrix in subsequent processing, resulting in exaggerated deformations. To address this issue, I crop out the excessively deformed areas after each stitching process.